BlueShark Series Electronic Water-proof Weighing Counter Scale

(LED Digital Display Storage Cell)

Operation Manual

Superior Technology

Excellent Performance

English in Brief

Catalogue

1 Cautions

1.1 Conventional cautions

1.2 Lead-acid battery attentions in use

2 Installation

2.1 Install scale’s body

2.2 Change battery

2.3 Turn on/off

3 Displays

3.1 Cursor indication

3.2 Function key

4 Operation

4.1 Set zero

4.2 Press the key to set zero

4.3 Direct Weighing

4.4 Tare

5 Battery Charging

6 Function Setting

7 Basic Configurations and Functions

8 Maintenance

Thanks for your choosing BlueSharp Series Electronic Water-proof Weighing Counter Scale of Fushun Yuchen Weighing Apparatus Co., Ltd. We hope our products’ excellent damp-proof and water-proof features and easy access can provide you the high efficiency.

1 Cautions

1.1 Conventional cautions

1. Please read the manual carefully before using the product so that you can understand the product’s functions exactly to smooth your operation.

2. Please keep scale’s body clean. Cut off the electricity before cleaning the body.

3. Clean the scale’s body with water and don’t put the scale into the water.

4. Please don’t reverse the scale or move it with the bracket in order not to destroy the sensor.

5. Please don’t touch the ultrathin seal ring between bracket and upper lid with sharp utensil so as not to poke the sealing.

1.2 Lead-acid Battery Attentions in Use

1. Our company’s lead-acid battery is maintenance free. It has low self-releasing rate and long life. Working temperature is -10℃ ～+40℃. Cycle charging and releasing times is 300 at 20℃. Time of charging is related to the remaining electric quantity. The more electricity remains, the less time of charging needs. Battery reaches the full charge for 15 hours charging after it releases completely.

2. Lead-acid battery’s cathode and anode can’t be short circuit or be connected reversely; otherwise it will be destroyed and lead to fire. The right way is: red wire to red battery plug and black wire to black battery plug.

3. Please keep the battery away from oil, water and any other chemical solvent since they can affect the battery’s life. Please don’t disassemble or repair the battery by yourself. Keep the battery away from fire.

4. Please charge the battery in time after it is exhaustive. Take out the battery if it is going to be left unused for an extended period of time to avoid battery leaking to destroy the scale and the battery should be fully charged for storing. Battery is charged at least once every three months.

5. Battery needs charging and releasing more than three times repeatedly if long time no use. When you first use the scale, the battery may not last for long, which is not the battery’s fault. Battery will back to normal after several times of charging and releasing.

6. Since lead-acid battery is an easy consumption part, its life is connected with user’s using method. (battery is excluded from maintenance)

7 With the increasing times of using battery, its releasing capacity will decrease after charging, that is to say, the working time of battery is shorter than before, which is the feature of charging battery. Please change a new battery if it needs recharge shortly after charging.

2 Installation

2.1 Install scale’s body

1. Open the package box, the contents as follows.

Number                 Name and Specification                     Unit       Quantity

1                 BlueShark Water-proof Weighing Scale               set           1

2                        Operation Manual                         share         1

3                     Product Qualification Certificate                 share         1

4                       After-sales Service Manual                   share          1

5                            Adapter                              piece         1

6                       Stainless Steel Scale Pan                     piece         1

2. Put scale on firm level surface and adjust scale’s feet (namely rotate scale’s feet). Adjust the water bubble of horizontal indicator on the panel inside of the central circle to keep the scale steady.

3. Install the pan. If use AC, please insert one plug of power cable into AC hub at the scale bottom and the other plug into electrical outlet. Press the ON key to start and go into weighing state.

2.2 Change battery

1 Loosen the screw at the scale bottom with cross screwdriver, then take down the lid of battery’s box and take out the battery.

2 Change new battery. Please don’t reverse the electrode. Cathode and anode can’t be short circuit.

Notice: Cathode and anode of lead-acid battery can’t be short circuit or reversed; otherwise that will destroy the battery and lead to fire. The right operation method is: red wire to red battery plug and black wire to black battery plug.

3. Cover the lid of battery’s box. Lay the sealing ring in the proper place to insure the sealing quality.

2.3 Turn on/off

1 Turn on

Press ON/OFF key for one second to start up. Scale begins self-examination automatically, then displays 1—9 in sequence and all cursors, at last lightens the zero cursor entering into the weighing state.

Notice: Make sure no articles on the pan before switching on.

2. Turn off

In the weighing state, press ON/OFF key for one second until OFF appears.

3 Displays

Notice: Digital display color: 1. bright red 2.orange 3.blue 4.yellow 5.pure green

3.1 Cursor indication

AC cursor-----light when input AC current

g cursor-----indicate current display of weight unit is gram

lb cursor-----indicate current display of weight unit is pound

zero cursor-----indicate current display of weight is zero(scale is at zero point)

steady cursor-----indicate current weight is steady

net weight cursor----- indicate current display of weight is net weight

lower limit-----indicate current weight is under the setting range

qualified cursor-----indicate current weight is in the setting range

upper limit-----indicate current weight is above the setting range

Notice: Under the state of direct current weighing, the left margin of display window displays “L” and flickers, which represents the scale is in low battery (the battery still can last for a moment at this time). The battery needs charging immediately to avoid damage due to over-exhaustion; otherwise the scale will power off automatically in a while.

3.2 Function key

ON/OFF key-----in the state of power off, press this key to start up; in the state of power on, press this key to shut down until showing OFF.

FUNCTION key-----function setup

TARE key-----press this key to show zero weight when weight is within the range of tare. It displays net weight when net weight cursor lights.

Set zero-----press this key to show zero weight when weight is within the range of setting zero.

4 Operation

4.1 Set zero

Set zero when starting up

When starting up for a while, scale will show “Err-3” to present it can’t find zero point if it detects weight has been beyond the range(surpass its capacity). At this time, please remove the articles on the pan to make the scale return to the normal position.

4.2 Set zero

Valid effective at gross weighing status. Press SET ZERO key to back to gross zero when scale displays negative gross weight or under-load sign OL or non-zero gross weight value.

4.3 Direct weighing

1. Put the articles on the pan, and then scale shows the weight value directly.

2. Remove the articles, and then scale backs to zero.

4.4 Tare

1. Put the container or package (50g) on the pan, press TARE key, and net cursor lights to show 0.000

2. Put the articles (400g) that need weighing into the container or package to show net weight 0.400

3. Repeating first step can tare continually.

4. Take down the articles and package to show tare weight value -0.050

5. Press TARE key to back to the status of zero gross weight.

Notice: Reversed tare is forbidden. When the weight value of the articles on the pan is less than the tare weight, tare is forbidden.

5 Charging battery

Please charge the battery following the below steps:

- Unplug the protection cork of AC plug.

- Charging in the state of power on/off, insert one end of adapter output plug into charging socket (AC indication lamp flickers in the process of charging, then turns into continual light when scale is in full charge to show the power capacity is 90%, if charge another 2～3 hours, the electrical quantity to 100%)

- When finishing charging, please plug the protection cork to prevent AC plug from corrosion in moisture circumstances.

Notice: please charging in dry places

6 Function setting

Operator can revise the function setting to make the electronic scale’s function suit customer’s personal needs.

Press ON key to start up. When scale is in self-examination state, press FUNCTION key firmly and then self-examination is over, go into customer function setting.

Press SET ZERO key to choose sequentially

Press TARE key to choose parameter

- FN1 X  parameter initialization  1:no 2:yes (press SET ZERO key for two seconds for default initialization setting)

- FN2 X setting for power-saving mode  1:off  2:on

- FN3 X time setting for power-saving mode  1:15seconds 2:30seconds

- FN4 X after display close  1:on  2:off

- FN5 X display mode setting  1: continual quick change

2: continual medium change

3: continual slow change

4: change after stability

- FN6 X  decimal point shape setting  1：。 2：；

- FN7 X weighing mode setting  1:weak 2: medium 3:strong(choose parameter according to different vibration conditions)

- FN8 XXXXX  low limit value setting, press FUNCTION key to shift, press TARE key to input data

- FN9 XXXXX  high limit value setting, press FUNCTION key to shift, press TARE key to input data

- FN10X  high/low limit notice before stability/after stability 1:before stability 2:after stability

- FN11 X  buzz output setting (buzzer version)

1: no output

2: output when weight within the setting range

3: output when weight beyond the setting range

Display END1

Press FUNCTION key to save setting returning to weighing state

Notice: fault initialization data as follows

FN1-1(no)    FN5-1(quick)   FN9-1(0)

FN2-1(OFF)   FN6-1( 。)    FN10-1(before stability)

FN3-1(light)   FN7-1(low)    FN11-1(no output)

FN4-1(ON)    FN8-1(0)

7 Basic configurations and functions

| BlueShark series water-proof weighing counter scale |  |  |  |  |
| --- | --- | --- | --- | --- |
| Capacity | 1.5kg | 3kg | 6kg | 15kg |
| Division value | 0.1/0.2/0.5g | 0.2/0.5/1g | 0.5/1/2g | 1/2/5g |

| Technical Index | Description |
| --- | --- |
| Display digit | 6 bit LED |
| Operation temperature | -10℃～+40℃ |
| Tare range | Full weight, the weighing range of net weight decreases accordingly when tare weight exists. |
| Stainless steel pan dimension | 230×190mm |
| Product appearance dimension | 303×243×125mm |
| Power supply | 100-240V～50/60Hz or 6V/5Ah lead-acid battery of maintenance free |
| Charging current | 1.0A |

8 Maintenance

Thanks for your choosing Yuchen Electronic Weighing Scale. We would provide you excellent after-sale services and try our best to solve the problems instantly in the process of your using the product.

You will obtain free maintenance service in exclusively authorized maintenance center of Yuchen with valid invoice and guarantee of repair certificate in one year’s time limit of purchasing, if the products break down not due to man-made reasons.

Warning

We would be no responsibility for the consequences of violating the above regulations and wrong operations due to the user’s faults.

Please keep this “operation manual” carefully for checking up in order to use our products conveniently.

Please contact the authorized dealer in time if you have any questions in the course of operating the BlueShark Series Electronic Water-proof Weighing Counter Scale.

Yuchen Weighing Apparatus Co. Ltd

Weighing the World

Implementing standard         GB/T   7722-2005

Information in this document is subject to change without notice.

Fushun Yuchen Weighing Apparatus Co., Ltd reserves the explanation rights for the above stated information.

Fushun Yuchen Weighing Apparatus Co., Ltd

Address: Liuhe Street No. 6-2   Shuncheng District   Fushun Liaoning Province113006, China

Tel: +86-413-7728318   7723822

Fax: +86-413-7736412

P.O.: 113006

Website: http://www.yuchenweigh.com

Email: yuchengroup@yuchenweigh.com

Registered trademark of Fushun Yuchen Weighing Apparatus Co., Ltd

This manual selects botanical ink printing

Inner pages adopt environmental-friendly recycled paper

窗体顶端

窗体底端